	  CHEVROLET


	  


	 MODEL
	 YEAR(S)

	 FULL SIZE PICKUP (NEW BODY STYLE)
	 1999 -2002


	 KEY
	 T-HARNESS
	 IMMOBILIZER

	  N/A
	  GM-11
	  GM's PASSLOCK 2 ANTI-THEFT System, Requires 791 Bypass Module


	 PART 
	 COLOR
	 LOCATION
	 DIAGRAM

	 12 VOLT CONSTANT
	  RED (+)
	  IGNITION SWITCH HARNESS
	 

	 STARTER
	 YELLOW (+)
	 IGNITION SWITCH HARNESS
	 

	 STARTER 2
	 N/A
	 
	 

	 IGNITION 1
	 PINK (+)
	 IGNITION SWITCH HARNESS
	 

	 IGNITION 2
	 WHITE (+) See NOTE *5
	 IGNITION SWITCH HARNESS
	 

	 IGNITION 3
	 N/A
	 
	 

	 ACCESSORY/HEATER BLOWER 1
	 ORANGE (+)
	 IGNITION SWITCH HARNESS
	 

	 ACCESSORY/HEATER BLOWER 2
	 N/A
	 
	 

	 KEYSENSE
	 N/A
	 
	 

	 PARKING LIGHTS ( - )
	 N/A
	 
	 

	 PARKING LIGHTS ( + )
	 BROWN (+)
	 @ BCM, LIGHT BLUE plug, Pin B6, See NOTE *1
	 

	 POWER LOCK
	 LIGHT BLUE (TYPE A)
	 IN DRIVERS DOOR HARNESS, FRONT of FUSE BOX or @ BCM, BROWN plug, See NOTE *1
	 

	 POWER UNLOCK
	 WHITE (TYPE A)
	 IN DRIVERS DOOR HARNESS, FRONT of FUSE BOX or @ BCM, BROWN plug, See NOTE *1
	 

	 LOCK MOTOR WIRE
	 GRAY (+)
	 IN DRIVERS DOOR HARNESS
	 

	 DOOR TRIGGER
	 See NOTE *2
	 @ BCM, See NOTE *1 See DIAGRAM
	 29358_FULL-SIZE-PICKUP-(NEW-BODY-STYLE)_(-) NEGATIVE DOOR PIN ISOLATION CIRCUIT.pdf

	 DOMELIGHT SUPERVISION
	 TAN (-)
	 @ BCM, See NOTE *1
	 

	 TRUNK RELEASE
	 N/A
	 
	 

	 SLIDING POWER DOOR
	 N/A
	 
	 

	 HORN
	 BLACK (-)
	 @ BCM, BROWN plug, See NOTE *1
	 

	 TACH
	 WHITE
	 @ ECM, See NOTE *3
	 

	 WAIT TO START LIGHT
	 See NOTE *4
	 
	 

	 BRAKE
	 WHITE (+)
	 @ SWITCH ABOVE BRAKE PEDAL
	 

	 FACTORY ALARM DISARM
	  LIGHT GREEN (-)
	 IN DRIVERS KICK PANEL
	 

	 ANTI-THEFT
	 YELLOW (+), ORANGE/BLACK (-) and RED/WHITE (Not used)
	 GM's PASSLOCK 2 System, these wires exit the Ignition Switch Tumbler together and are in the Main Ignition Switch Harness in the Column.
	 


	NOTES

	 NOTE *1 The BCM (Body Control Module) is located behind the drivers side of dash. The plugs on the BCM face the rear of the vehicle.


NOTE *2 At the BCM the DRIVER DOOR is a TAN (-) Pin B4 of the PURPLE PLUG, the PASSENGER DOOR is a ORANGE (-) Pin B2. When connecting to an alarm system use both wires and DIODE ISOLATE See DIAGRAM. When connecting for DOMELIGHT SUPERVISION use the TAN (-) Pin B4 at the PURPLE Plug.


NOTE *3 The ECM (Engine Control Module) is located near the battery and down low between the fender and fan under a BLACK plastic cover. On the 4.3L engine the TACH wire in at Pin 20 in the connector marked "RED" On the 4.8L, 5.3L and 6.0L engines, it is located at Pin10 in the RED connector.


NOTE *4 when hooking up a REMOTE STARTER on a DIESEL, if the WAIT to START wire from the Remote Starter does not wait until the Light on the dash turn out before cranking, you will need to program in the 5,10 or 15 Second WAIT to START TIMMER in the Installation Manual. On some vehicles the Wait to Start Light on the dash is not a true (+)Positive or (-)Negative and the wire from the Remote Starter will not owrk. When this Wait to Start Timmer is programmed, disconnect and tape up the Wait to Start wire from the Remote Starter that is tied into the Wait to Start Light on the dash.


NOTE *5 IMPORTANT !!!!! this wire MUST BE CONNECTED as IGNITION #2 for REMOTE STARTING!!!!!!


