Service Information

Document ID: 4924668

#18-NA-001: Information On Headlamp Shakes or Flickers - (Jan 4, 2018)

| Subject: | Information On Headlamp Shakes or Flickers |  |
| --- | --- | --- |

| Brand: | Model: |  | Model Year: |  | VIN: |  | Engine: | Transmission: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | from | to | from | to |  |  |
| Chevrolet | Silverado Models |  | 2015 | 2018 |  |  |  |  |
| GMC | Sierra Models |  |  |  |  |  |  |  |
| Involved Region or Country |  | North America |  |  |  |  |  |  |
| Condition |  | Some customers may comment that the headlamp shakes or flickers at night over bumps. |  |  |  |  |  |  |
| Information |  | HID headlamps do not have filaments like traditional bulbs. Instead, high-voltage is used in order to create an arc between electrodes in the arc tube (bulb). On HD trucks, when driven on dirt roads or rough pavement, the impact of road irregularities changes the position of the arc inside the capsule located in the light housing. This causes the arc to move on the electrode changing the position within the capsule (bulb) body and the relationship between the arc, the reflector, and the lens of the lamp. This condition may be more noticeable on HD trucks, due to the stiffer suspension. The vehicle should be compared to a known good vehicle: - If they are the same, Do Not replace the headlamp assemblies, bulbs, or any other component of the headlamp system for this condition. The light has not failed, it has no effect on the longevity of the system, there is no safety issue and only occurs at the edge of the pattern on rough road surfaces. - If the vehicle in question is notably worse than a comparable unit, the vehicle should be inspected for anything that might cause the lamp housing to be unstable and repairs should be made to correct that condition only. |  |  |  |  |  |  |

Parts Information

No parts are required for this repair.